### Elektrický PEUGEOT 106
možnost výběru z následujících dvou verzí:

4-místné osobní vozidlo

2-místné vozidlo (montážní verze)

Cena vozidel: nová vozidla stojí (bez baterií) cca 19.000 USD, naše cena – včetně baterií (!) je cca 200.000 Kč!!! Vozidla mají najeto cca 30 tis. km. Možná návštěva a nezávazné možnost vyzkoušení. Dále je k dispozici i montážní verze Peugeot Partner.

#### Design and performance data

Parametry vozidla*:

| Design data: |  |
| --- | --- |
| Model designation: | Peugeot 106 Electrique |
| Physical dimensions and weights |  |
| Length: | 3.56 m |
| Width: | 1.59 m |
| Height: | 1.37 m |
| Front track: | 1.38 m |
| Rear track: | 1.30 m |
| Luggage space, 4 seat model | 0.18/0.38 m3 |
| Luggage space, 2 seat model | 0.88 m3 |
| Cargo space: | 3.0 m3 |
| Kerb weight (Weight of complete vehicle with a full tank of heater fuel): | 1087/1077 kg |
| Gross vehicle weight , 4 seat model | 1375 kg |
| Gross vehicle weight , 2 seat model | 1367 kg |
| Payload: | 285 kg |
| Drive train |  |
| Electric traction motor, source: | Leroy-Somer |
| Type | DC, seperately exited, liquid cooled |
| Peak power: | 20 kW |
| Nominal power: | 11 kW |
| Max. torque: | 127 Nm |
| Driving modes, reduction gear and transmission |  |
| Drive selector modes: | Forward, reverse, neutral |
| Reduction gear ratio: | 7.2 |
| Traction: | Front wheel drive |
| Traction batteries |  |
| Source: | SAFT |
| Type: | NiCd |
| Nominal voltage: | 120 V |
| Capacity: | 100 Ah |
| Onboard charger (intergrated with electronic control unit) |  |
| Max continuous power: | 3 kW |
| Cable with connector for charging |  |
| Cable length: | 5 m |
| Connector, vehicle side: | Maréchal plug |
| Connection, grid side: | Single phase, 230 V, 16 A |
| Interior heating and ventilation systém |  |
| Heat source: | Petrol combustor |
| Source: | Webasto |
| Max power: | 5 kW |
| Max fuel consumption: | 0.7 litre/hour |
| Fuel tank capacity: | 4 litres |
| Performance data |  |
| Maximum speed: | 90 km/h |
| Acceleration time, 0 – 50 km/h: | 8.5 seconds |
| Range according to CEN TC 301 N36: | 90 km |
| Energy conversion rate: | 0.20 kWh/ton km |

KONTAKT A INFORMACE:  EKOLO | Sedlec-Prčice | Komenského 275 | tel.317 834 230

mob. 606 846 717 | SMS 608 400 000 | Email:ekolo@seznam.cz	| elektromobil@tiscali.cz

* (v angličtině; údaje převzaty z projektu ZEUS – „Zero and low Emiission in Urban Society“ )